Ergonomics

Hi. My name’s Anastacia Sax. I’m going to discuss such an important topic as Ergonomics. (2)On the agenda we have 3 lesson objectives: Identify common work-related musculoskeletal disorders (MSDs); Recognize risk factors associated with work-related MSDs; and Identify ergonomic control methods for reducing work-related MSDs.

(3)Ergonomics is the scientific discipline concerned with understanding of interactions among humans and other elements of a system, and the profession that applies theory, principles, methods and data to design in order to optimize human well-being and overall system performance. It’s in charge of designing jobs, equipment, and work tasks to fit human physical characteristics and energy limitations. It considers body dimensions, mobility, and the body's stress behavior.

(4)As they say, "make the work fit the person, not the person fit the work".

(5)Why is Ergonomics important?

Overexertion is the leading cause of injuries. It takes about 35% of Workers' Compensation expenses. Moreover, persistent pain may develop in the future.

Bodily reaction (like bending, climbing, reaching, standing, sitting) is another leading cause of injuries in the workplace.

Repetitive motion is also within top 10 most common workplace accidents.

(6)Work-Related MSDs

Musculoskeletal disorders (MSDs) affect the muscles, nerves, blood vessels, ligaments and tendons. Workers in many different industries can be exposed to risk factors at work, such as lifting heavy items, reaching overhead, pushing and pulling loads or working in awkward body postures. Sure, it increases a risk of injury. Fortunately, Ergonomics helps to lessen muscle fatigue and increases productivity by that.

(7)Most commonly affected areas are back, arms, shoulders, neck, hands, knees, and feet.

(8)Risk factors

The risk of MSD injury depends on work positions and postures, how often the task is performed, the level of required effort and how long the task lasts. (9)Risk factors that may lead to the development of MSDs include:

Exerting excessive force. For example, lifting heavy objects, pushing or pulling, manually pouring materials, or maintaining control of equipment.

Performing the same motion or series of motions continually or frequently for an extended period of time.

Working in awkward postures or being in the same posture for a long time. Using positions that place stress on the body, such as repetitive reaching above shoulder height, kneeling, squatting, leaning over a counter, using a knife with wrists bent, or twisting the torso while lifting.

(10)Localized pressure into the body. Pressing a body part against hard or sharp edges, maybe using the hand as a hammer.

Cold temperatures. In combination with any of the above risk factors may also increase the potential for disorders. For instance, many of the operations in meatpacking occur with a chilled product or in a cold environment.

Vibration. Both whole body and hand-arm, can cause a number of health effects. Hand-arm vibration can damage small capillaries that supply nutrients and can make hand tools more difficult to control. The effects of vibration can damage the body and greatly increase the force which must be exerted for a task.

Combined exposure to several risk factors.

(11)Control methods

And to avoid these unwanted factors Ergonomics designed special control methods:

Training is an important element in the ergonomic process. Training should be conducted in a vocabulary that all workers understand and is best provided by individuals who have experience with ergonomic issues. When training is effective workers will learn ergonomic principles and applications, learn about the proper use of tools and machine controls, use good work practices including proper lifting techniques, become aware of work tasks leading to pain.

Recognize early symptoms. It means understanding the importance of reporting and addressing early indications of disorders before serious injuries and illnesses develop.

(12)Job Hazard Analysis. Where a work-related MSD exists, all employees performing a similar job must be observed and the job evaluated using either one or more of the job hazard analysis tools. Once ergonomic risks have been identified the employer must implement controls to eliminate the risks. This table usually includes 3 columns that can be used while conducting. The first column provides a list of tasks performed by a job; the middle one is provided for identified hazards; and, the third one provides control measures to reduce the dangers.

(13)Repetitive motions

Let’s take a look at how Ergonomics solves the repetitive motion issue.

High repetition involves the same movements for at least 2 hours at a time;(14) or, using a high repetition device (e.g. keyboard, mouse) for more than 4 hours a day.

Fewer people know that repetitive lifting can be just as hazardous as heavy lifting. When you lift frequently you tire out the muscles, making them more prone to injury. If your muscles can't handle the load, the strain can be shifted to your joints and spine.

Repetitive motions may be required by your job. However, a lot of times you have some control over what motions you make and how often.

(15)Things you can do to reduce repetitive motions include:

Arrange work to avoid unnecessary motions

Let machinery do the work

Spread repetitive work out during the day

Take stretch pauses

Rotate tasks with co-workers if possible

Change hands and motions frequently

Conclusion

(16)In summary, here are 5 key points to remember about ergonomics:

1.Ergonomics can help you on your job. It’s a tool you’re able to use to make your job safer and better. Washington state even has a rule that requires employers to implement ergonomics in the workplace.

2.MSDs can happen in jobs with risk factors.

3.By applying ergonomics, risk factors can be reduced and MSDs prevented.

4.If you start having symptoms of an MSD, be sure you report them early to get rid of a more serious injury.

5.You’re the expert when it comes to your job, and you can play an important role in your company's ergonomics efforts. Ergonomics can be more than just a one-time fix. It can be an on-going process that you and your employer can use to make things better.

(17)Thanks for your attention!